# Project Goal

To design a feedback controller of quadrotor drone. To simplify the model, the rotor and propeller properties will not be considered. Pulling force generated from propellers will be considered input. Moreover, wind effect will not be taken into consideration. Finally, only headless mode will be implemented in this model, where no yaw will be conducted by this drone.

# Model Definition

Figure 1 The definition of axis and variables. Image from Douglas et. al. 2016.

Figure 2 Explanation how yaw generated in drone. Figure from Sabatino, 2015

Sabatino described the dynamics of the actuators as Equation 1, where ft is the total force generated; Ωi is the velocity of the propeller i; b is the ratio of pulling force generated based on angular velocity of propellers; d is the drag coefficient of propellers.

Equation 1 Actuator dynamics from 4 motors. Sabatino, 2015

To simplify this model, I select to use Headless mode of drone, where the direction of the drone is fixed. That is, no rotation along Z axis, which can be express as ψ=0, ∀t 
τz=0 
ψ=0, ψ=0, ψ=0
So the force in Equation 1 can be simplified as:

ft=F1+F2+F3+F4
τx=l(F2-F4)
τy=l(F3-F1)
F2+F4=F1+F3

Equation 2 Simplified Actuator dynamics from 4 motors given no Z axis rotation.

Also, to simply the model, wind effect is not considered in this report.

So the system dynamics can be simplified as follows, in Equation 3:

- where pink line crossed variables are simplified because of ψ=0, r=ψ=0, ψ=0.

- where green line crossed variables are simplified because wind factors are not considered.

- blue box indicates the input to the system.

Equation 3 The quadrotor system dynamics equation defined based on
state variable that X=ψ  θ  φ  p  q  r  u  v  w  x  y  z. Sabatino, 2015.
c for cos(); s for sin(); t for tan().

To summarize, the simplified the overall dynamics of the headless drone system with no wind effect can be concluded as follows:

φ=p+rcφtθ+qsφtθ⋯⋯⋯⋯⋯⋯  (Eq4.1)θ=qcφ-rsφ⋯⋯⋯⋯⋯⋯⋯⋯⋯⋯⋯   (Eq4.2)p=τxIx⋯⋯⋯⋯⋯⋯⋯⋯⋯⋯⋯⋯⋯⋯⋯⋯⋯⋯  (Eq4.3)q=τyIy⋯⋯⋯⋯⋯⋯⋯⋯⋯⋯⋯⋯⋯⋯⋯⋯⋯⋯  (Eq4.4)x=-θ∙z-g∙s(θ)⋯⋯⋯⋯⋯⋯⋯⋯⋯⋯⋯⋯   (Eq4.5)y=φ∙z+g∙c(θ)s(φ)⋯⋯⋯⋯⋯⋯⋯⋯⋯⋯⋯  (Eq4.6)z=θ∙x-φ∙y+g∙cθcφ-ftm⋯⋯⋯⋯⋯⋯  (Eq4.7)

Equation 4 The headless quadrotor dynamics summarized with no wind effect.

Define the states variables as X=x  y  z  x  y  z  φ  θ  p  q, the input is defined as  u=[ft  τx  τy]. Then the nonlinear state space system can be described as Equation 5:

X=fX+g1Xft+g2Xτx+g3Xτy

fX=xyz-θ∙z-g∙s(θ)φ∙z+g∙c(θ)s(φ)θ∙x-φ∙y+g∙cθcφp+rcφtθ+qsφtθqcφ-rsφ00

g1X=[00000-1m0000]

g2X=000000001Ix0

g3X=0000000001Iy

Linearization

An equilibrium point is found by set X=0:

From Eq 4.6, sφ=0→ φ=0

From Eq 4.5, cφsθ=0→ θ=0

From Eq 4.7, z=g-ftm=0→g=ftm→ ft=mg

From Eq 4.3, p=τxIx=0→ τx=0

From Eq 4.4, q=τyIy=0→ τy=0

Therefore, the equilibrium point is X=x  y  z  0  0  0  0  0  0  0, where x,y,z ∈R,

and u=[mg  0  0]

The linearization can be calculated as follows:

A=∂f(x,u)∂x=00010000000000100000000001000000000-θ0-g0-z00000φg0z00000θ-φ00-yx0000000010000000000100000000000000000000

At the equilibrium point, with the assumption that when θ is small, sinθ→θ,cosθ→1:

AX=0=0001000000000010000000000100000000000-g00000000g00000000000000000000010000000000100000000000000000000

B=∂f(x,u)∂u=0000000000000001m0000000001Ix0001Iy

Output Definition

The output that can be measured form the system is from accelerometer, gyroscope and barometer. The angular velocity φ  θ can be measured by gyroscope. The z displacement z can be measured by barometer. The velocity in x and y direction x  y can be measured by integration of accelerometer signals, though noisy.

Therefore C=00000000100000000001001000000000010000000000100000 and D=000000000000000

Summary

The system has 3 inputs: u=ftτx τy.

The system has 10 states: X=x  y  z  x  y  z  φ  θ  p  q

The system has 5 outputs: y=φ  θ  z  x  y

(handout #2 3.1)

The Jordan form of the system is: (handout #2 3.2)

J=0100000000001000000000010000000000000000000001000000000010000000000100000000000000000000010000000000

The eigenvalues are all zero. The Jordan form consists of 2 blocks with zero eigenvalues of size 4 (blue and green), and one block with zero eigenvalues of size 2 (red).

Therefore, the system is not stable. (handout #2 3.3).

The nonlinear system is not stable at equilibrium point, any disturbance can make the drone fall. (handout #2 3.4)

The system is not BIBO stable because of zero poles. (handout #2 3.5)

I forgot to bring the lipo battery with me to Canada and it is extremely difficult to buy proper one. I was not able to do a test on a real system. (handout #2 3.6)

The open loop simulation shows the linear drift of the linear system and an exponential drift in the nonlinear system. (handout #2 3.7)

Figure 3 Open loop simulation of the drone system. Exponential deviation from equilibrium point can be observed on the nonlinear system (orange line).

Step in inputs is not interesting since this system is extremely unstable. (handout #2 3.8)

Step in Fm:

Figure 4 The step response in total pulling force. The linear and nonlinear system go to opposite directions.

Step in τx

Figure 5 A step torque input was applied on x direction. A constant increase of angular velocity can be observed in x direction.

Step in τy

Figure 6 A step torque input was applied on y direction. A constant increase of angular velocity can be observed in y direction.

Because the response of the nonlinear system is so large, the response of the linear system is neglectable. (handout #2 3.9)

As a drone, the controller should be able to make drone stay in a relative stable position in the air. It should not drift around. At least, the controller should not make the drone fall on the ground. (handout #2 3.10)

The tentative specification can be defined as follows:

- The drone should be able to take off and land.

- The drone should be able to stay in certain height with limited drift, within 0.1m/s.

- The drone should be able to move up to 3m/s.

The system is controllable. Controllability matrix ranks 10, the same as number of states. (handout #3 2.1)

The input of the system is simplified to the total pulling force and torque generated by the propellers. So, one motor fail will impact all three inputs. If we only consider the impact on torque, the system is already uncontrollable (Controllability matrix ranks 6<10). (handout #3 2.3)

The Kalman decomposition result is:

Ak=00000000000001000000-100000000000000000000-9.8000000000000-1000000000000010000000-9.800000000000000000000-1.414200 Bk=10.04990.00010000-0000-0.04990-0-1000000-00000000000000

Ck=0+0-0-10000+0000000000+10000.99500000+0000000001000000100000

Dimension of CO=5, CO=1, CO=3, CO=1.

The 4 dimensions where the uncontrollable have zero eigenvalues. So, the system is not stabilizable if one motor is broken. (handout #3 2.3)

The controller is design with LQR, where the Q=I and R=0.0001. The response if quick and match the specification defined above (handout #3 2.4). The drone can fly and hold its position in air. The top speed reached around 6m/s.

Figure 7 The LQR controller quickly balance the drone back to equilibrium point and the only drift left is on the z axis because of the gravity.

I was not able to design a proper controller by eigenvalue placement. The best result is as follows:

Figure 8 The eigenvalue placement controller can balance the drone back, but not to the equilibrium point.

The rotation angle in x and y direction, instead of converging back to zero, it stabilized at multiple of 2pi. And a very sharp oscillation can be observed in multiple states. (handout #3 2.5)

With the LQR controller, the initial condition was quickly compensated, where the initial condition is:

- 0.06m for x direction offset

- -0.01m for y direction offset

- No z offset

- 0.008m/s in x direction

- 0.017m/s in y direction

- 0.04m/s in z direction

- -0.006 rad in x rotation direction

- -0.008 rad in y rotation direction

Figure 9

Even if the initial condition is amplified for 5 times, it is also quickly compensated. It is a strong controller. (handout #3 2.6)

Figure 10

Noise is added to the states. The noise in the translational directions is expected to be small, since the position of drone can be measured by indoor camera systems. And the rotational noise is also small, since multiple IR reflectors can be used for precise angle measurement.

- The linear noise in position is assumed to be 1cm

- The linear noise in velocity is assumed to be 1cm/s

- The rotational noise in angles is assumed to be 1 degree

- The rotational noise in angular velocity is assumed to be 1 degree/s

With noise in the states, the LQR controller performs as the same. Probably the noise is small. (handout #3 2.7)

The eigenvalue placement controller can also handle the noise. It seems that the eigenvalue placement controller is more sensitive to initial conditions than noise. This probably because of the eigenvalue I assigned are smaller than those generated by LQR, so the drone takes more time to stable given nonzero initial conditions.

The drone system is not observable, the rank of observability matrix is 8<10. The x and y position is not observable with barometers, accelerometers and gyroscope. (handout #4 2.1).

The Kalman decomposition is:

Ak=-0.0050-0100-000-00-0-00-0-9.810000+0000+09.81000000-0-00.00500-000-0000-00-010000+0+00-00+010+000000000000+000-00+00-0+00--100-00+00+0000100-00000 Bk=0-00-0-00-0001000+0-000-10000000-100000+00000000

Ck=000000-1000000000-0-100-0.99500-0.00500+00000-1000+0+000000-10000000

The system is not detectable, because the unobservable part has zero eigenvalues.

To make the drone x and y direction position detectable, a GPS sensor can be added, e.g. A9G GPRS/GPS module. (handout #4 2.2).

An observer was design based on the linearization system. The linear observer is used to estimate the states in the nonlinear system. The setup of “a linear observer on a linear system” is not implemented, since the system exactly match, and the controller response is so quick. The simulation would be meaningless since the would match perfectly.

The linear observer got a good estimation on x or y position, exact match with the nonlinear system. The angles estimation is reasonable. A constant deviation was observed in z direction before adding a PID output controller. After that the z direction deviation as diminished. (handout #4 2.4&2.5)

A tracking route was added for the drone. The drone is expected to raise slowly, and draw circles in x and y plain, 1m radius, 10s per round:

- Z=tanh(t)

- X= Z*sind(t*36)

- Y= Z*cosd(t*36)

The simulation result is as follows: (handout #4 2.6)

The z axis has an overshoot, and x y direction have a phase lag when compared with the tracking target.

Assume a disturbance as wind making the drone to drift in x direction, from T=5 to T=6:

Assume a disturbance as wind making drone rotate in x rotation direction, from T=5 to T=6:

We can see the disturbance is instantly compensated.

Assume a disturbance as wind making the drone to drift in z direction, from T=5 to T=6:

# Reference

Tesch, Douglas A., Diego Eckhard, and William Cechin Guarienti. "Pitch and roll control of a quadcopter using cascade iterative feedback tuning." IFAC-PapersOnLine 49.30 (2016): 30-35.

Sabatino, Francesco. "Quadrotor control: modeling, nonlinearcontrol design, and simulation." (2015).